Radar Site Selection for PSR, SSR and SMR at Bandaranaike International Airport (BIA), Katunayake, Sri Lanka

- Georeferencing

- Digitization

- PostgreSQL/PostGIS Database Development

- KML/KMZ Data Collection from Google Earth

- QGIS Integration

- Buffer Analysis/intersection/difference

- Overlay Analysis

- Suitability Analysis

- Building Count and Area Calculations

- GPS Coordinate Extraction

- Final Map Preparation

- Discussion and Findings

Objective

Develop a fully georeferenced digital aerial map of Bandaranaike International Airport (CMB) and its surrounding suburbs using QGIS. The map should identify the most suitable locations for deploying three radar systems:

- Primary Surveillance Radar (PSR)

- Secondary Surveillance Radar (SSR)

- Surface Movement Radar (SMR)

The project should include GIS analysis, spatial database management, and a final professional map layout.

Task 1 – Georeference and digitization of the Aerial Image (Mandatory)

Before digitizing any features, you must georeference the provided aerial image.

Requirements

- Use the QGIS Georeferencer tool.

- Assign the Coordinate Reference System (CRS):

  - EPSG:5234

- Use suitable Ground Control Points (GCPs).

- Produce a georeferenced raster.

Output:

- Georeferenced aerial image.

Task 2 – Create a Spatial Database

SL_BIA_Aerial_Info

The database should store all spatial and attribute data created during the project.

It should contain multiple layers.

Example layers:

- Buildings

- Roads

- Airport Boundary

- Runways

- Taxiways

- Water Bodies

- Vegetation

- Open Land

- Radar Suitability Areas

- GPS Locations

Task 3 – Digitize Features

Digitize all visible features from the aerial image.

Examples:

- Buildings

- Roads

- Runways

- Taxiways

- Airport Fence

- Open Land

- Water Bodies

- Trees

- Railway

- Parking Areas

Every vector layer must contain the following fields:

| Field | Data Type |
| --- | --- |
| id | Integer |
| name | Text |
| type | Text |
| size | Decimal |

Example

| id | name | type | size |
| --- | --- | --- | --- |
| 1 | Passenger Terminal | Building | 8500 |
| 2 | Runway 04/22 | Runway | 3500 |
| 3 | Hangar A | Building | 2100 |

Task 4 – Extract GPS Locations

Use Google Earth Pro.

Create KML/KMZ files containing GPS coordinates for:

- Proposed PSR locations

- Proposed SSR locations

- Proposed SMR locations

Import the KML/KMZ files into QGIS.

Task 5 – Spatial Analysis

Using QGIS geoprocessing tools, identify the most suitable radar deployment areas.

Possible analysis includes:

- Buffer analysis

- Clip

- Intersect

- Dissolve

- Select by Location

- Union

- Difference

- Overlay analysis

Consider factors such as:

- Open land

- Distance from buildings

- Distance from runway

- Clear line of sight

- Airport operational area

- Accessibility

Task 6 – Identify Suitable Radar Locations

The final map should clearly indicate suitable locations for:

Surface Movement Radar (SMR)

-SMR system should be installed at an appropriate location approximately 300 m from the CMB  control tower

-about 200 m from the Runway Center Point (RCP )

Requires:

- Close to runway and taxiways

- Good visibility of aircraft movements

- Airport operational zone

Secondary Surveillance Radar (SSR)/PSR

-The PSR and SSR systems should be positioned at a suitable site approximately 2 km from the RCP, while remaining within a maximum of 3 km from the RCP

-PSR and SSR installations should ideally be located within the Sri Lanka Air Force base premises

Task 7 – Perform Spatial Calculations

The assignment specifically asks you to calculate:

1. Total Number of Buildings

Determine how many buildings are located within the selected radar suitability area(s).

Use:

- Select by Location

- Count Features

2. Total Land Area Occupied by Buildings

Calculate:

Sum of all building footprints inside the suitability area.

Use:

Field Calculator

$area

Then calculate the total.

3. Total Available Land for Radar Deployment

Calculate

Suitable Area

minus

Building Area

minus

Restricted Area

Result

Available land for deployment.

Task 8 – Produce an Informative Map

Suitable Radar Deployment Areas around

Bandaranaike International Airport (CMB)

Map Elements

- North Arrow

- Scale Bar

- Legend

- Coordinate Grid

- CRS (EPSG:5234)

- Labels

- Airport Boundary

- Roads

- Buildings

- Proposed Radar Locations

- Suitability Areas

Expected Outputs

1. Georeferenced aerial image

2. Spatial database

SL_BIA_Aerial_Info

containing all vector layers.

3. Digitized GIS layers

- Buildings

- Roads

- Airport boundary

- Runways

- Taxiways

- Radar suitability areas

- GPS points

4. Imported GPS points

Using KML/KMZ files from Google Earth.

5. Spatial analysis results

Showing suitable areas for:

- PSR

- SSR

- SMR

6. Statistical results

Including:

- Total number of buildings within suitability areas.

- Total building area within suitability areas.

- Total land area available for radar deployment.

7. Final professional map layout

A fully referenced, labeled, and informative aerial map with all required GIS elements, suitable for presentation or submission.

Steps

- Georeference the aerial image (EPSG:5234).

- Create the SL_BIA_Aerial_Info spatial database.

- Digitize all required features into separate vector layers with the required attribute fields (id, name, type, size).

- Extract candidate radar locations in Google Earth and import them as KML/KMZ into QGIS.

- Perform spatial analysis (buffers, overlays, clipping, intersections) to identify suitable PSR, SSR, and SMR deployment areas.

- Calculate the required statistics: number of buildings, total building area, and remaining available land.

- Design a professional map layout including legend, scale, north arrow, coordinates, CRS (EPSG:5234), labels, and the identified radar deployment locations.

PostgreSQL/PostGIS Database Creation

Install

- PostgreSQL

- PostGIS

Create Database

- Using pgAdmin

- Create database

CREATE DATABASE SL_BIA_Aerial_Info;

Enable PostGIS:    CREATE EXTENSION postgis;

Connect PostgreSQL with QGIS

Browser Panel --- PostgreSQL  → New Connection

Host = localhost

Port = 5432

Database = SL_BIA_Aerial_Info

User = postgres

Password = xxxx

Connect

Import Layers into PostgreSQL

In QGIS: Right Click Layer

Export → Save Features As

Select;  PostgreSQL

Target: SL_BIA_Aerial_Info

Tables; eg:

buildings

roads

runways

taxiways

airforce

radar_sites

Radar Suitability Analysis

SMR

300 m from Control Tower

200 m from RCP

Create Buffers

Control Tower - Buffer = 300 m

RCP - Buffer = 200 m

Vector → Geoprocessing → Buffer

Intersection - Find common area

Vector → Geoprocessing → Intersection

You will get, SMR_Suitability_Zone

PSR/SSR Suitability

≈2 km from RCP/ Maximum 3 km

Create Buffers

RCP:Buffer: 2000 m  and 3000 m

Difference Operation ; Create ring zone:

3000m Buffer minus 2000m Buffer

PSR_SSR_Ring

Restrict to Air Force Base

Intersection between PSR_SSR_Ring and SLAF Boundary

Output : PSR_SSR_Suitable_Area

Calculate Number of Buildings

Select by Location

Appendices

Appendices Task a

Appendices Task b